# m52221demo quick start guide

### Introduction and Default Settings

This quick start guide will show how to connect the target board to a PC, install the correct version of CodeWarrior Development Studio, and execute a simple demonstration program loaded in FLASH memory.  The default jumper settings for the M52221DEMO are shown below

Figure 1: M52221DEMO Default Settings

### Install CodeWarrior Development Studio for ColdFire Special Edition

To install CodeWarrior Development Studio for ColdFire Special Edition, refer to the “CodeWarrior Development Studio” DVD case and follow the steps in the included Quick Start Guide.

CodeWarrior Development Studios must be properly installed before attempting to connect the target board to the host PC.  Otherwise, the necessary USB drivers will not be available and the target board will not be recognized when connected to the host PC.

### Run the demo program

The M52221DEMO ships with a simple demonstration program preloaded into on-chip FLASH memory.  The source code for this demo program is located on the Axiom Support CD in the /Examples folder.

- Verify the configuration jumpers are set to the default position.  Refer to Figure 1 above.

- Connect the included A/B USB cable between an open USB port on the host PC and the USB connector on the target board.  Follow the on-screen instructions to install the necessary USB drivers.

- After the USB drivers are properly installed, the USB, POWER OUT, and +5V LED’s will be ON.

- Connect the 6-in-1 USB cable between and open USB port on the host PC and the mini-AB connector on the target board.

- Power-cycle the board.  The on-screen cursor will oscillate back and forth.