DEMO9S08SE8

Lab Tutorials

MCU Port Connector Pin Out

The following list shows the pin out for the MCU port connector on the DEMOSE8 board (J1).

| VX | 1 | 2 | PTA5 /IRQ/TCLK/RESET |
| --- | --- | --- | --- |
| VSS | 3 | 4 | PTA5 /IRQ/TCLK/RESET |
| PTB1 /KBIP5/TxD/ADP7 | 5 | 6 | PTA4 /BKGD/MS |
| PTB0 /KBIP4/RxD/ADP6 | 7 | 8 | PTB7 /EXTAL |
| PTA2/KBIP2/ADP2 | 9 | 10 | PTB6/XTAL |
| PTA3/KBIP3/ADP3 | 11 | 12 | NA |
| PTA0/KBIP0/TPM1CH0/ADP0 | 13 | 14 | NA |
| PTB6/XTAL | 15 | 16 | NA |
| PTB3/KBIP7/ADP9 | 17 | 18 | PTA1/KBIP1/TPM1CH1/ADP1 |
| PTB4/TPM2CH0 | 19 | 20 | PTA0/KBIP0/TPM1CH0/ADP0 |
| PTB2/KBIP6/ADP8 | 21 | 22 | NA |
| PTB5 | 23 | 24 | NA |
| PTC0 | 25 | 26 | PTB1/KBIP5/TxD/ADP7 |
| PTC1 | 27 | 28 | PTB0 /KBIP4/RxD/ADP6 |
| PTC2 | 29 | 30 | PTA6/TPM1CH0/ADP4 |
| PTC3 | 31 | 32 | PTA7/TPM1CH1/ADP5 |
| PTC4 | 33 | 34 | PTC7 |
| PTC5 | 35 | 36 | PTC6 |
| NA | 37 | 38 | NA |
| NA | 39 | 40 | NA |

DEFAULT JUMPER SETTINGS

The following list shows the default jumper settings for DEMOSE8 board.

| Jumper | Status |
| --- | --- |
| PWR_SEL | VB |
| VX_EN | installed |
| JP301,RST,BGND | installed |
| JP302,GPIO,DATA1,DATA2 | installed |
| COM_SEL | COM |
| USER1,SW1,SW2, LED1,LED2,RV1,RZ1 | installed |

LAB1

This document explains how to set up and use the SE8_DEMO_LAB1 example application. The example application allows the user to try out several features on Freescale’s MC9S08SE8 microcontroller.

Setting up the Demo

1. Connect the DEMO9S08SE8 demo board to the PC with USB cable. The jumper configuration for this application is:

– SW1, SW2, LED1, LED2, RV1, RZ1 jumpers are installed

– COM_SEL switches to COM

2. Connect the DB9 serial cable between the DEMO9S08SE8 and the PC.

3. Open both the CodeWarrior IDE and the SE8_DEMO_LAB1 project.

4. To compile for the P&E ICD target and program the MCU,select “P&E Multilink/Cyclone Pro” from the pull-down menu beneath the project name. Then click the debugger icon. This will compile the project of SE8_DEMO_LAB1, launch the debugger application and prompt user with “Erase and program FLASH”. Click on “Yes”. The MCU is now programmed.

5. After the MCU is programmed, launch the terminal emulator. The terminal emulator must be configured to use the COM port that the DB-9 cable is plugged into, usually COM1. The COM port settings must be:

•115200 baud rate

•8 bits

•No parity

•1 stop bit

The demo is now ready to run.

Running the Demo

Now press the “RUN” button. If everything is configured correctly, the terminal window will display the following as soon as the demo starts:

Welcome to the World of MC9S08SE8 Demo Application

V1.0

POR LVD, Reset Occurred

Main Test Menu:

[1] Real Time Clock setup

[2] TPM1 PWM setup

[3] ADC test

[4] GO STOP

Please select a number to execute:

- Bus clock source

This demo relies on the internal clock reference to generate bus frequency and the communications baud rate. Therefore, the MCU’s internal clock reference has been trimmed to 32 kHz.

- System Reset Status

After every chip reset, the terminal will display the System Reset Status and state the cause or causes of the last reset.

- RESET Button

For this demo application, the RESET pin on the MC9S08SE8 is configured as an IRQ pin function. The user can use the IRQ function to wake it up from the low power mode.

- Potentiometer RV1

On the demo board, the RV1 is a potentiometer. The user can turn the RV1 to get the voltage (from 0V to 5V) on the RV1 jumper.

- Photo sensor RZ1

On the demo board, the RZ1 is a photo sensor. The voltage of RZ1 jumper varies from 0V to 5V if the intensity of light on the sensor RZ1 varies. The darker the photosensor (RZ1), the higher is the voltage. Usually in the room environment, the voltage of RZ1 jumper is about 5V, so the user needs a light source, such as lamp, to get lower voltages of RZ1 jumper.

Demo Menu Options

The user can choose from four options to explore a few of the MC9S08SE8’s features and go to the submenu of each function.

To make a selection, simply type the number next to the menu item.

[1] Real Time Clock setup

Real Time Clock Menu:

[1]: Set up, enable and display Real Time Clock

[3]: Disable Real Time Clock

[e]: Exit Real Time Clock menu

Please select a number to execute:

This menu option lets the user enable, disable and exit the Real Time Clock which is the basis of RTC module.

When number 1 is selected, the clock is enabled and displayed on the terminal after the current time input. Please enter the current time according to the display: Enter current time (format: HHMM): (H-hour, M-minute). The clock will be displayed on the terminal.

When number 3 is selected, the RTC is disabled,and the clock disappears from the terminal.

[2] TPM1 PWM setup

TPM1 Menu:

[1]: Enable TPM1

[2]: Disable TPM1

[e]: Exit TPM1 menu

Please select test number to execute:

This option lets the user enable or disable the PWM function of TPM1.

When number 1 is selected, channel 1 of TPM1 which is connected to LED1 is configured to central-aligned PWM. The LED1 will be illuminated. You can press the SW1 to adjust the luminance of the LED1. When number 2 is selected, TPM1 is turned off.

[3]ADC Menu:

[1]: Enable  ADC to measure the RZ1

[2]: Disable ADC

[e]: Exit ADC menu

Please select test number to execute:

When number 1 is selected, ADC is enabled to sample the output of RZ1. If the voltage on the photosensor (RZ1) is higher than 4V, LED2 will be lit up. If the RZ1 voltage is less than 4V, LED2 will not be lit up. The voltage of RZ1 jumper varies from 0V to 5V if the intensity of light on the sensor RZ1 varies. The darker the photosensor (RZ1), the higher is the voltage. In normal room environment, the voltage of RZ1 jumper is about 5V, so the user needs a light source, such as lamp, to get the lower voltages of RZ1 jumper. When number 2 is selected, ADC is turned off.

[4] STOP test

GO to Stop Menu:

[2]: Go to Stop2

[3]: Go to Stop3

[e]: Exit Stop menu

Please select test number to execute:

The “STOP test” options allow the user to configure the MCU for either stop2 or stop3 mode.

When number 2 is selected, MCU goes to STOP2 mode as configured. The terminal displays: Stop2 Selected.

In this demo code, there are only two ways to wake up the MCU: RTC and IRQ. If the user enables RTC, MCU can wake up from both STOP2 and STOP3 modes.

When number 3 is selected, MCU goes to STOP3 mode as configured. The terminal displays: Stop3 Selected.

There are several ways to wake up the MCU from STOP3 mode. The IRQ could be used to wake it up from the STOP3 mode by press the reset pin. Another way to wake up MCU from STOP3 is RTC. If the user enables clock, every minutes the time is updated, which could wake up MCU from both STOP2 and STOP3 mode. When it is waken up from Stop3 mode the terminal displays: Wake up from STOP3.

When 2 pressed, MCU goes to STOP2 mode as configured. The terminal displays: Stop2 Selected. In this demo code, only 2 ways could be used to wake up the MCU: RTC and IRQ. If the user enables clock, every minutes the time is updated, which could wake up MCU from both STOP2 and STOP3 mode.